Computer Networks - Homework 2 Answers

Spring 2026  |  2452865 邹世豪

Question 1: Basic Functions and Interfaces

The data link layer (Layer 2) sits between the physical layer and the network layer. It turns raw bit transmission into reliable, addressed, hop-by-hop frame delivery.

Framing: Encapsulates network-layer packets into frames with a header and trailer, so the receiver can detect frame boundaries.

Physical/MAC addressing: Adds source and destination MAC addresses to each frame to identify the sender and next-hop receiver on the local link.

Error detection: Appends a CRC checksum to each frame; the receiver recomputes it and discards any corrupted frame.

Flow control: Prevents a fast sender from overwhelming a slow receiver, using mechanisms such as sliding windows.

Medium access control: On shared media, the MAC sublayer arbitrates channel access (e.g., CSMA/CD for Ethernet, CSMA/CA for Wi-Fi).

Interface with the physical layer: The data link layer passes a bit stream down to the physical layer for transmission and accepts raw bits from it on reception. It is transparent to the physical medium type.

Interface with the network layer: The network layer hands an IP datagram to the data link layer, which wraps it in a frame and delivers it one hop. At the destination, the data link layer strips the frame and passes the datagram up to the network layer.

Question 2: CRC Encoding Calculation

Given: data D = 1101011011, generator G(x) = x³ + x + 1, binary divisor = 1011 (degree r = 3).

Step 1: Append r = 3 zero bits to D to get the augmented dividend:

D' = 1101011011 000  (13 bits)

Step 2: Perform modulo-2 long division of D' by G = 1011. Each step XORs the working 4-bit group with 1011 when the leading bit is 1, or with 0000 when the leading bit is 0 (shift only).

1101011011000

1011

----

1101  (XOR 1011) = 0110, bring down 1 → 1100

1100  (XOR 1011) = 0111, bring down 1 → 1111

1111  (XOR 1011) = 0100, bring down 0 → 1000

1000  (XOR 1011) = 0011, bring down 1 → 0111 → 1110

1110  (XOR 1011) = 0101, bring down 0 → 1010

1010  (XOR 1011) = 0001, bring down 0 → 0010

0010  (leading 0, shift)  bring down 0 → 0100

Remainder = 100  (3 bits)

CRC checksum (FCS) = 100

Transmitted bit stream = 1101011011 100

Verification: dividing 1101011011100 by 1011 yields remainder 000, confirming the encoding is correct.

Question 3: Selective Repeat and GBN Protocols

Selective Repeat (SR) — schematic (window W = 4, frame 1 lost):

Sender:    [0][1][2][3] ------>  frame 1 lost  -----> retransmit [1] only

Receiver:  ACK 0; buffer 2, 3; wait for 1; after 1 arrives, deliver 1-2-3 in order

Go-Back-N (GBN) — schematic (window N = 4, frame 1 lost):

Sender:    [0][1][2][3] ------>  frame 1 lost  -----> retransmit [1][2][3]

Receiver:  ACK 0; discard 2 and 3 (out of order); wait for 1

Comparison:

| Aspect | Selective Repeat (SR) | Go-Back-N (GBN) |
| --- | --- | --- |
| Design goal | Maximise efficiency by retransmitting only the lost or corrupted frame. | Simplify implementation with cumulative ACKs and strictly in-order delivery. |
| Receiver window | Equal to sender window (≤ 2^(n−1)); buffers out-of-order frames. | Always 1; any out-of-order frame is immediately discarded. |
| ACK type | Individual ACK per correctly received frame. | Cumulative ACK for the last in-order frame received. |
| Retransmission | Only the specific missing frame is retransmitted. | On timeout, the missing frame and all subsequent frames in the window are retransmitted. |
| Sequence numbers | Need at least 2W distinct values (window ≤ 2^(n−1)). | Need at least W+1 distinct values (window ≤ 2^n − 1). |
| Buffer cost | Receiver must buffer out-of-order frames; higher memory cost. | No receiver buffer needed; simpler but wastes bandwidth on errors. |

Question 4: Stop-and-Wait Protocol Performance

Given: R = 100 Mbps, one-way propagation delay Tp = 20 ms, frame length L = 1000 B = 8000 bits. ACK size is assumed negligible.

Transmission time:  Tt = L / R = 8000 / (100×10⁶) = 0.08 ms

Round-trip time:    RTT = 2 × Tp = 40 ms

Cycle time:         Tt + RTT = 0.08 + 40 = 40.08 ms

Utilisation:        U = Tt / (Tt + RTT) = 0.08 / 40.08 ≈ 0.001995 ≈ 0.2%

Max throughput:     U × R = 0.001995 × 100 Mbps ≈ 199.5 Kbps

The utilisation is extremely low because the propagation RTT (40 ms) is 500× larger than the transmission time (0.08 ms). A sliding-window protocol with window size W ≥ RTT/Tt ≈ 501 frames would be needed to fully utilise the link.

Question 5: CSMA/CD Working Principle

CSMA/CD (Carrier Sense Multiple Access with Collision Detection) is the MAC protocol used in classic shared Ethernet. It works in four stages:

Carrier sense: Before transmitting, a station listens to the channel. If the channel is busy it waits; if idle it begins transmitting immediately.

Collision detection: While transmitting, the station monitors the channel voltage. If the detected signal differs from what it is sending, a collision is inferred.

Jam and abort: On detecting a collision, the station transmits a 32-bit jam signal to ensure all stations recognise the collision, then stops transmitting.

Binary exponential backoff: Each colliding station waits a random number of slot times drawn from {0, …, 2^k − 1} after the k-th collision (capped at k = 10), then retries. After 16 failures the frame is dropped.

Suitability for wireless networks: CSMA/CD is NOT suitable for wireless networks, for two fundamental reasons.

(1) Cannot detect collisions: A wireless transmitter's own signal overwhelms any received signal, making it physically impossible to detect a superimposed collision while transmitting.

(2) Hidden terminal problem: Station A and station C may both be out of each other's range but both transmit to B simultaneously. A cannot sense C's carrier, so carrier sense alone cannot prevent the collision at B.

Wi-Fi therefore uses CSMA/CA (Collision Avoidance): random backoff before transmission, virtual carrier sensing (NAV timer), and optional RTS/CTS handshake to reduce collisions without requiring collision detection.

Question 6: Ethernet Collision Detection

Given: bus length d = 1 km, propagation speed v = 2×10⁸ m/s, data rate R = 1 Gbps.

One-way propagation delay:  τ = d / v = 1000 / (2×10⁸) = 5 μs

Part 1 — Minimum frame length

A sender must still be transmitting when the collision signal returns, i.e., the frame transmission time must be ≥ the worst-case round-trip time 2τ.

Minimum frame size = R × 2τ = 10⁹ bps × 10×10⁻⁶ s = 10 000 bits = 1250 Bytes

Part 2 — Maximum time for A to detect the collision

In the worst case A and B are at opposite ends of the bus. A’s signal takes τ = 5 μs to reach B; the collision signal then travels τ = 5 μs back to A.

Maximum detection time = 2τ = 10 μs after A starts transmitting.

Part 3 — Can A detect the collision before finishing a 1500-Byte frame?

Frame size:          1500 B = 12 000 bits

Transmission time:   Tt = 12 000 / 10⁹ = 12 μs

Collision detected:  at most 2τ = 10 μs after A starts

Since 10 μs < 12 μs, A is still transmitting (2 μs remain) when the collision is detected. Yes, A can detect the collision before finishing the frame.

Question 7: Switch Self-Learning and Forwarding

An Ethernet switch builds its MAC table by self-learning: on every incoming frame it records (source MAC → ingress port). It forwards based on destination MAC using that table.

Step 1 — A sends to B: Switch learns A → port A. Destination B is unknown → flood to all ports except port A.

Step 2 — B sends to A: Switch learns B → port B. Destination A is now known → forward only to port A (unicast, no flooding).

Step 3 — C sends to A: Switch learns C → port C. Destination A is known → forward only to port A.

| Event | MAC table state | Forwarding decision |
| --- | --- | --- |
| A → B | A → port A | B unknown: flood to all ports except port A |
| B → A | A → port A; B → port B | A known: forward only to port A |
| C → A | A → port A; B → port B; C → port C | A known: forward only to port A |

Table entries age out after a timeout (typically 5 minutes). If a host moves to a different port, the old entry is replaced when the host next sends a frame.

Question 8: IPv4 Datagram Fragmentation

Given: total length = 4000 B, IP header = 20 B, payload = 3980 B, MTU = 1500 B.

Max payload per fragment = MTU − header = 1500 − 20 = 1480 B  (divisible by 8 ✓)

Number of fragments = ⌨12 3980 / 1480 ⌓ = 3

Part 1 — Fragment details:

| Fragment | Payload (B) | Total length (B) | Fragment Offset | MF bit |
| --- | --- | --- | --- | --- |
| 1 | 1480 | 1500 | 0 / 8 = 0 | 1 |
| 2 | 1480 | 1500 | 1480 / 8 = 185 | 1 |
| 3 | 1020 | 1040 | 2960 / 8 = 370 | 0 |

Part 2 — If fragment 2 is lost:

The receiver cannot reassemble the original datagram. IPv4 reassembly requires all fragments to be present; it places each fragment’s payload at the byte offset given by Fragment Offset×8. With fragment 2 missing, bytes 1480–2959 of the original payload are absent and the gap cannot be filled. After a reassembly timer expires the destination discards all received fragments and sends an ICMP Time Exceeded (fragment reassembly timeout) message to the source. Recovery is left to the transport layer (e.g., TCP retransmission).

Question 9: Shortest Path Calculation

Link weights (ms): A–B = 2, A–C = 5, A–D = 3, C–D = 2, B–E = 1, D–E = 3, B–F = 3, E–F = 1.

Part 1 — Dijkstra’s algorithm from source A:

| Step | Visited node | Cost fixed | Distance updates |
| --- | --- | --- | --- |
| 0 | A | A = 0 | B = 2, C = 5, D = 3, E = ∞, F = ∞ |
| 1 | B | B = 2 | E = min(∞, 2+1) = 3; F = min(∞, 2+3) = 5 |
| 2 | D | D = 3 | C = min(5, 3+2) = 5; E unchanged (3 < 3+3) |
| 3 | E | E = 3 | F = min(5, 3+1) = 4 |
| 4 | F | F = 4 | No updates |
| 5 | C | C = 5 | Done |

| Destination | Shortest path | Cost |
| --- | --- | --- |
| B | A → B | 2 ms |
| C | A → C (or A→D→C) | 5 ms |
| D | A → D | 3 ms |
| E | A → B → E | 3 ms |
| F | A → B → E → F | 4 ms |

Part 2 — B–F reduced from 3 ms to 1 ms:

A → B → F:       2 + 1 = 3 ms  (new)

A → B → E → F: 2 + 1 + 1 = 4 ms  (old shortest)

Yes, the shortest path changes. The new shortest path from A to F is A → B → F with cost 3 ms.

Question 10: IPv4 Subnetting

Original network: 192.168.10.0/24. Requirement: ≥ 6 subnets.

Borrow 3 host bits: 2³ = 8 subnets  ≥  6  ✓

New prefix length = /24 + 3 = /27

Addresses per subnet = 2^(32−27) = 32;  usable hosts = 32 − 2 = 30

| No. | Network address | Usable host range | Broadcast | Hosts |
| --- | --- | --- | --- | --- |
| 1 | 192.168.10.0/27 | 192.168.10.1 – .30 | 192.168.10.31 | 30 |
| 2 | 192.168.10.32/27 | 192.168.10.33 – .62 | 192.168.10.63 | 30 |
| 3 | 192.168.10.64/27 | 192.168.10.65 – .94 | 192.168.10.95 | 30 |
| 4 | 192.168.10.96/27 | 192.168.10.97 – .126 | 192.168.10.127 | 30 |
| 5 | 192.168.10.128/27 | 192.168.10.129 – .158 | 192.168.10.159 | 30 |
| 6 | 192.168.10.160/27 | 192.168.10.161 – .190 | 192.168.10.191 | 30 |
| 7 | 192.168.10.192/27 | 192.168.10.193 – .222 | 192.168.10.223 | 30 |
| 8 | 192.168.10.224/27 | 192.168.10.225 – .254 | 192.168.10.255 | 30 |

Question 11: Router Forwarding Table

Routers use longest-prefix matching (LPM): when a destination address matches multiple prefixes, the one with the greatest prefix length wins.

| Destination address | Matching prefixes | Longest match | Next hop |
| --- | --- | --- | --- |
| 172.16.10.5 | /16, /21, /24 | 172.16.10.0/24 | R3 |
| 172.16.9.100 | /16, /21 | 172.16.8.0/21 | R2 |
| 172.16.20.1 | /16 only | 172.16.0.0/16 | R1 |

172.16.10.5 → R3: 3rd octet = 10 matches the /24 prefix exactly.

172.16.9.100 → R2: 3rd octet = 9 (00001001). The /21 mask (255.255.248.0) checks the top 5 bits of the 3rd octet: 00001, which matches 172.16.8.0 (00001000). The /24 does not match (octet 9 ≠ 10).

172.16.20.1 → R1: 3rd octet = 20 (00010100). Top 5 bits = 00010 ≠ 00001, so /21 does not match. Only the /16 matches.